CERTIFICATE OF QUALITY AND QUANTITY NoLT2120-1-167-738-QW

In pursuance to an order received from: our Principals to carry out instructions summarized as:

| GOODS DESCRIPTION | : | POTASSIUM CHLORIDE (KALI CLORUA LONG HAI (MOP)) |
| --- | --- | --- |
| PACKING | : | in 1000bags jumbo bags in 23x20’DRY containers |
| BILL OF LADING NUMBER | : | HDMUKLJA13729500 dated 25.10.2021 |
| VESSEL AND VOYAGE №. | : | BERNHARD SCHEPERS voy. 0281N |
| TOTAL NET WEIGHT | : | 552.000 MT |
| PORT OF LOADING PLACE OF INSPECTION | : : | Klaipeda, Lithuania BKT Terminal, Klaipeda Port, Lithuania |
| PORT OF DISCHARGE | : | Hochiminh, Vietnam |
| DATE OF INSPECTION | : | 14-18.10.2021 |

This is to report that in accordance with instructions received from: our Principals we have carried out inspection of a parcel of POTASSIUM CHLORIDE (KALI CLORUA LONG HAI (MOP)) between 14-18.10.2021.

Quality:

At the request of our principal, we have collected representative samples, intended for control of quality and for retention. Samples were taken manually during loading and prepared according to Fertiliser Directive ECC 2003/2003 dd. 13.10.2003. Sampling accuracy is limited to the method of sampling available in the time and place of inspection.

One composite sample was submitted to SGS Klaipeda Ltd. laboratory for analyses.

Based on the analysis effected and within the limits of the results obtained from laboratory we certify our findings as follows:

| TESTS | METHODS | UNITS | LIMITS | RESULTS |
| --- | --- | --- | --- | --- |
| K2O | *GOST 20851.3 method 2 | mass % | min. 60.0 | 60.6 |
| Potassium chloride (KCl) water soluble | calculated | mass % | min. 95.0 | 96 |
| Moisture | *GOST 20851.4 method 2 | mass % | max. 0.50 | 0.25 |
| Colour | visual |  | white | white |
| Sieve test | *ISO 8397 |  |  |  |
| below 2.00 mm |  | mass % | min. 90.0 | 99.9 |

The goods are free flowing, and free from extraneous harmful substances at time of loading, according to visual inspection.

Quantity:

Determination of NET weight by means of re-weighting. GROSS weight is calculated by adding of marked empty container weight.

Total net weight loaded 552.000 MT (23 containers).

Total gross weight loaded 553.265 MT.

| No. | Container No. | Container weight Net, kg | Cargo weight Net, kg | Weight Brutto of bags, kg | Tally of bags | Container Line's seal No.: |
| --- | --- | --- | --- | --- | --- | --- |
| 1 | KOCU2233899 | 24055 | 24000 | 1003 | 24 | 1778 |
| 2 | CAIU3885135 | 24055 | 24000 | 1003 | 24 | 1779 |
| 3 | KOCU2021267 | 24055 | 24000 | 1003 | 24 | 1780 |
| 4 | HDMU2621821 | 24055 | 24000 | 1003 | 24 | 1781 |
| 5 | KOCU2183864 | 24055 | 24000 | 1003 | 24 | 1782 |
| 6 | BSIU2359580 | 24055 | 24000 | 1003 | 24 | 1783 |
| 7 | GCXU2110843 | 24055 | 24000 | 1003 | 24 | 1784 |
| 8 | KOCU2074502 | 24055 | 24000 | 1003 | 24 | 1785 |
| 9 | HMMU2042495 | 24055 | 24000 | 1003 | 24 | 1786 |
| 10 | TLLU2591927 | 24055 | 24000 | 1003 | 24 | 1787 |
| 11 | GCXU2125988 | 24055 | 24000 | 1003 | 24 | 1788 |
| 12 | BEAU2036139 | 24055 | 24000 | 1003 | 24 | 1789 |
| 13 | HDMU2690165 | 24055 | 24000 | 1003 | 24 | 1790 |
| 14 | HMMU2056122 | 24055 | 24000 | 1003 | 24 | 1791 |
| 15 | KOCU2012033 | 24055 | 24000 | 1003 | 24 | 1792 |
| 16 | BMOU2904760 | 24055 | 24000 | 1003 | 24 | 1793 |
| 17 | TRHU2164825 | 24055 | 24000 | 1003 | 24 | 1794 |
| 18 | HDMU2851315 | 24055 | 24000 | 1003 | 24 | 1795 |
| 19 | TCKU3596450 | 24055 | 24000 | 1003 | 24 | 1796 |
| 20 | KOCU2232017 | 24055 | 24000 | 1003 | 24 | 1797 |
| 21 | KOCU2112562 | 24055 | 24000 | 1003 | 24 | 1799 |
| 22 | HDMU2871965 | 24055 | 24000 | 1003 | 24 | 1798 |
| 23 | TTNU1192056 | 24055 | 24000 | 1003 | 24 | 1800 |
| Total: | 23 Containers | 553265 | 552000 |  | 552 |  |

This certificate reflects our findings at time and place of our intervention only and does not relieve the parties from their contractual responsibilities.

| Signed and dated at Klaipeda port October 25, 2021 |  | For and on behalf of SGS Klaipeda Ltd. Surveyor |
| --- | --- | --- |